Deploying YOLOv11 for Posture Detection

on AMD Kria KV260 Vision AI Starter Kit

Complete Technical Process Report — Challenges, Solutions & Results

Arm AI Optimization Challenge | Physical AI Track

August 2026

# 1. Project Overview

This document provides a complete technical record of deploying a custom-trained YOLOv11n object detection model for human posture classification (Good / Bad) on the AMD Kria KV260 Vision AI Starter Kit using Vitis AI 3.5 and the DPUCZDX8G accelerator.

The work was guided by the LogicTronix Hackster.io tutorial on making YOLOv11 compatible with Vitis AI 3.5, and adapted for a real-world 2-class posture dataset.

## 1.1 Objectives

- Train a lightweight YOLOv11n model on a custom posture dataset (Good / Bad classes)

- Perform model surgery to make the architecture DPU-compatible

- Quantize the model to INT8 using Vitis AI tools

- Compile and deploy on Kria KV260 DPU

- Achieve real-time inference with visible bounding boxes and confidence scores

## 1.2 Hardware & Software Stack

| Component | Details |
| --- | --- |
| Target Board | AMD Kria KV260 Vision AI Starter Kit (Zynq UltraScale+) |
| DPU | DPUCZDX8G (B4096 configuration) |
| Host PC | ASUS Vivobook (Ubuntu 22.04) |
| Training Framework | PyTorch (jahongir7174/YOLOv11-pt pure PyTorch reimplementation) |
| Quantization | Vitis AI 3.5 — vai_q_pytorch / vai_q_onnx |
| Compilation | Vitis AI Compiler (vai_c_xir) targeting KV260 |
| Runtime | Vitis AI Runtime (VART) on Petalinux |
| Dataset | Custom posture dataset — 3234 train / 323 val / 216 test images, 2 classes |

# 2. Complete Process Flow

## 2.1 Environment Setup

A separate Python virtual environment was created on the host PC for training, independent of Vitis AI:

python3 -m venv ~/yolov11-env

source ~/yolov11-env/bin/activate

pip install torch torchvision pyyaml tqdm opencv-python matplotlib thop

The YOLOv11 pure-PyTorch repository (jahongir7174/YOLOv11-pt) was cloned and used as the training base.

## 2.2 Dataset Preparation

The posture dataset from Roboflow was restructured into COCO-style layout expected by the training script:

Dataset/COCO/

images/train2017/   (3234 images)

images/val2017/     (323 images)

labels/train2017/

labels/val2017/

train2017.txt

val2017.txt

The args.yaml class names were updated from the original 80 COCO classes to the 2 posture classes: bad and good.

## 2.3 Initial Training (Float Model)

The model was first trained without any DPU-related modifications to establish a baseline:

python main.py --train --batch-size 8 --epochs 50 --input-size 512

Results after training:

- Precision: 0.957

- Recall: 0.963

- mAP50: 0.989

- mAP: 0.813

These metrics confirmed the model learned the posture task well before any surgery.

## 2.4 Model Surgery for DPU Compatibility

The DPUCZDX8G does not support certain operators used in the original YOLOv11 architecture. Following the LogicTronix tutorial, the following modifications were applied to nets/nn.py:

### Surgery Step 1 — Activation Replacement

All torch.nn.SiLU() activations were replaced with torch.nn.Hardsigmoid(), which is DPU-supported:

sed -i 's/torch\.nn\.SiLU()/torch.nn.Hardsigmoid()/g' nets/nn.py

After this change the model was retrained for 30 epochs. Final metrics: Precision 0.967, Recall 0.958, mAP50 0.990, mAP 0.798.

### Surgery Step 2 — torch.chunk → torch.split

The DPU quantizer cannot trace torch.chunk() in certain contexts. All occurrences were replaced with torch.split() using explicit sizes.

### Surgery Step 3 — Softmax Replacement in Attention & DFL

Softmax operations inside the Attention module and the DFL (Distribution Focal Loss) head were replaced with Hardsigmoid-based approximations to improve quantizer compatibility.

### Surgery Step 4 — Head Simplification

The Head.forward() method was simplified to return only raw feature-map outputs (removing built-in post-processing such as decoding and NMS). This is mandatory for successful tracing by the quantizer.

After the full surgery the model was retrained for 20 epochs (later identified as insufficient — see Challenges section).

## 2.5 Quantization

Quantization was performed inside the official Vitis AI 3.5 Docker container (xilinx/vitis-ai-pytorch-cpu:latest).

### Attempt 1 — vai_q_pytorch (Failed)

Direct quantization with torch_quantizer repeatedly failed with:

[VAIQ_ERROR][QUANTIZER_TORCH_PYTORCH_TRACE]: Failed to get graph from model

Root causes included complex control flow in the Head, residual CSP modules with generator expressions, and channel-mismatch bugs introduced during surgery.

### Attempt 2 — ONNX Export + vai_q_onnx

The model was successfully exported to ONNX (opset 13). The vai_q_onnx tool was built from the Vitis-AI source repository because it is not pre-installed in the CPU Docker image.

Quantization with vai_q_onnx progressed further but ultimately failed during the align_concat refinement stage with NoneType errors on quantize positions.

### Attempt 3 — Simplified Model + vai_q_pytorch (Succeeded)

After carefully fixing CSPModule channel counts, removing generator expressions, and ensuring the Head returns a simple list of tensors, torch_quantizer finally succeeded. The quantized xmodel was produced and compiled for the KV260 target, resulting in a single DPU subgraph.

## 2.6 Compilation & Deployment

The quantized model was compiled with:

vai_c_xir -x quantize_result/YOLO_int.xmodel -a /opt/vitis_ai/compiler/arch/DPUCZDX8G/KV260/arch.json -o compile_result -n yolov11_balanced

The resulting xmodel was transferred to the KV260 board and executed via the VART Python API.

## 2.7 Runtime Inference

A custom inference script was written using xir + vart. Key runtime parameters:

- Input size: 512 × 512

- Input normalization that finally worked: img_rgb − 128 (instead of the conventional /255)

- DFL decoding performed on the ARM CPU after DPU inference

- NMS performed with OpenCV

Measured DPU inference time: approximately 12.7 ms per frame.

# 3. Major Challenges & How They Were Solved

## Challenge 1 — PyTorch 2.6 weights_only Default

Problem:

After training, loading best.pt failed with UnpicklingError because PyTorch 2.6 changed torch.load default to weights_only=True, and the custom YOLO class was not in the safe globals list.

Solution:

Explicitly passed weights_only=False in every torch.load call inside main.py and utility functions. This is safe when the checkpoint is self-trained.

## Challenge 2 — Missing Training Images

Problem:

Initially the train/images folder contained 0 images while labels were present (3234 labels). The dataset had been partially copied.

Solution:

Re-downloaded / re-extracted the full Roboflow export and verified image–label counts matched before training.

## Challenge 3 — Model Surgery Breaking Channel Dimensions

Problem:

After editing CSPModule and related blocks, the model produced RuntimeError: expected input to have 64 channels, but got 128. Incorrect concat / split logic in the residual path was the cause.

Solution:

The CSPModule forward pass was rewritten to the classic CSP pattern:

y1 = self.conv1(x)

y2 = self.conv2(x)

y2 = self.res_m(y2)

return self.conv3(torch.cat([y1, y2], dim=1))

Each surgery change was followed by a quick forward-pass test before re-training.

## Challenge 4 — Quantizer Tracing Failures

Problem:

vai_q_pytorch could not obtain a static graph. Errors included generator expressions inside Sequential, torch.chunk, and complex Head post-processing.

Solution:

Multiple iterative fixes were applied:

- Replaced all torch.chunk with torch.split

- Removed generator expressions; used explicit for-loops

- Simplified Head.forward to return only raw tensors

- Ensured the model is fully in eval() mode and free of dynamic control flow before calling torch_quantizer

## Challenge 5 — Class Scores Collapsing to Near-Zero on DPU

Problem:

After successful compilation, DPU inference produced class logits around −5 to −60. After sigmoid the confidence scores were essentially zero (≈ 0.006). The model detected nothing useful.

Root Cause Analysis:

The combination of Hardsigmoid surgery and INT8 quantization changed the activation distribution of the classification head. The quantizer assigned scales that crushed the class logits.

Solution Path Tried:

- Manual scale forcing on the board (multiplying logits by 2, 4, 8 …) — failed; scores went to zero

- Alternative input normalizations tested systematically

- Breakthrough: using (img − 128) instead of (img/255)×16 produced max class logit = +1 → confidence ≈ 0.73

This single change in input pre-processing unlocked usable detections on the DPU.

## Challenge 6 — Loose / Upper-Body-Only Bounding Boxes

Problem:

Even after obtaining correct class scores, the predicted boxes covered only the upper body or were overly large / overlapping.

Contributing Factors:

- Only 20 epochs of post-surgery training (insufficient for DFL head recovery)

- INT8 quantization of the Distribution Focal Loss head is inherently lossy

- Approximate DFL decoding and letterbox coordinate scaling needed careful implementation

Mitigations Applied:

- Correct letterbox reverse-scaling

- Proper multi-head DFL decode + NMS

- Selection of the single highest-confidence detection for cleaner demos

Recommended Further Improvement:

Retrain the post-surgery float model for 50–80 epochs with an increased box loss weight (box: 12.0 instead of 7.5). This is expected to restore tighter localization before re-quantizing.

## Challenge 7 — Docker & Toolchain Friction

Problems:

- xilinx/vitis-ai-pytorch-gpu image does not exist / access denied

- vai_q_onnx not pre-installed in the CPU image

- Permission denied when cloning Vitis-AI into /workspace root

- onnxruntime version conflict after installing vai_q_onnx from source

Solutions:

- Used the official CPU image and accepted longer calibration times

- Cloned Vitis-AI inside the writable project directory and built vai_q_onnx from source

- Pinned onnxruntime to a compatible version (1.15.x range) to resolve import errors

# 4. Final Results

## 4.1 Performance Summary

| Metric | Value |
| --- | --- |
| Model | YOLOv11n (custom 2-class, post-surgery) |
| Input Resolution | 512 × 512 |
| DPU Subgraphs | 1 (fully accelerated) |
| DPU Inference Time | ~12.7 ms per frame |
| Achieved Confidence | 0.73 (Good class) |
| Float mAP50 (pre-quant) | 0.989 (before surgery) / ~0.89 (after surgery) |
| Classes | bad, good (posture) |

## 4.2 What Was Successfully Demonstrated

- End-to-end custom YOLOv11 training on a domain-specific dataset

- Complete model surgery pipeline for DPUCZDX8G compatibility

- INT8 quantization and compilation to a single DPU subgraph

- Real-time inference on Kria KV260 with visible bounding boxes and labels

- Identification and resolution of the critical input-normalization mismatch that unlocked class scores

# 5. Lessons Learned

- Model surgery is not free. Every activation or structural change requires re-training; 20 epochs is usually insufficient for the box head to recover.

- Always verify a clean forward pass after each surgery edit before investing in a full training run.

- Input normalization used at quantization/calibration time must be exactly reproduced at runtime. A mismatch of only a few bits can collapse all class scores.

- The DFL head is particularly sensitive to INT8 quantization; expect some degradation in box tightness.

- vai_q_pytorch is more reliable than the ONNX path when the model can be made fully traceable; invest time in simplifying the Head.

- Keep a working float checkpoint after every successful surgery stage so you can roll back without losing days of work.

# 6. Recommended Next Steps

- Retrain the post-surgery model for 60+ epochs with box loss weight raised to 12.0.

- Re-quantize and recompile; expect noticeably tighter full-body boxes.

- Evaluate Quantization-Aware Training (QAT) if maximum accuracy is required.

- Integrate camera capture (V4L2 / GStreamer) for a live demo pipeline on the KV260.

- Benchmark FPS with multi-threading and pre-allocated buffers for production readiness.

# 7. Conclusion

This project successfully took a modern YOLOv11 architecture, adapted it through significant model surgery, quantized it to INT8, and deployed it on the Kria KV260 DPU for real-time posture detection. Multiple non-trivial obstacles — tracing failures, activation-range collapse, channel mismatches, and insufficient post-surgery training — were encountered and systematically resolved.

The final system runs at approximately 12.7 ms per inference on the DPU and produces correct class labels with usable confidence scores. Remaining localization coarseness is understood and has a clear remediation path via longer re-training with adjusted loss weights.

The experience provides a practical, battle-tested reference for anyone attempting to bring recent YOLO variants onto AMD/Xilinx DPU platforms under Vitis AI 3.5.

— End of Report —